LAKE KASHGAWIGMOG WATER TESTING

Date:   August 14, 2019

Time:  0930 - 1230

Air Temperature: 22C

Water Temp at control site: 	23C

Secchi Disk Reading at Control site:  4.95 M.

| Site# | Site Location | Raw Cloriform | Coliform MPN Per 100 ml sample | Raw Ecoli | EColi MPN per 100ml Sample |
| --- | --- | --- | --- | --- | --- |
| 1 | Drag River at Grass L |  | 271 |  | 25 |
| 2 | Bonnie view/Halimar |  | 247 |  | 114 |
| 3 | Wigamog |  | 161 |  | 30 |
| 4 | Marcus Beach |  | 79 |  | 3 |
| 5 | Soyers L. Bridge |  | 102 |  | 19 |
| 6 | Control site 130’ depth |  | 16 |  | <3 |
| 7 | Ingoldsby 200’ from bridge |  | 39 |  | 5 |
| 8 | Chateau Woodland |  | 72 |  | 5 |
| 9 | Drag River at Head Lake |  | 166 |  | 13 |
| 10 | Northern Eagle |  | 151 |  | 3 |

Typical August Readings, not as high as last year tho.  Note Northern Eagle instead of Willow Bch

Readings of 1000 Plus for Coliform/or 100 Plus for Ecoli should be retested and if result verified – action should be taken as the water is unsafe for swimming